Rose-Hulman Institute of Technology

Authors: Elena Chong Loo

Lingtongyue Jin

ECE310: Final Laboratory Report

“Infrared Digital Communication System”

Submitted to: Professor Yong Jin Kim

Submission Date: October 13th, 2014

Fall 2014

Executive Summary

The goal of the first five weeks of laboratory for the ECE310 course was to build an infrared digital communication system from scratch capable of transmitting and receiving data produced by a pseudorandom data generator. This provided the students a better understanding of how each subsystems are used together to allow an efficient communication of information.

This digital communication system was completed successfully by the authors. The verification of the system was done using the equipment in lab B200 bench 4. The breadboard, including all the electronic components were bought in the school part room. The functionality of the transmitter and receiver were verified by sending the pseudorandom data from the transmitter and receiving the same signal in the receiver sider. The verification was done using an oscilloscope by visualizing the pseudorandom data at the transmitter and the receiver. Both devices were placed at 45 inches away, which was the maximum spacing over which the circuit continues to operate properly.

- Introduction

  - Project Overview

This project was developed through the laboratory requirement of the ECE310 course. The infrared digital communication system, which consists of a transmitter and a receiver circuit was built by the authors throughout the first five weeks of required laboratory. Two breadboards and electronics components, such as resistors, capacitors, op amps, etc., were bought from the part room at the school and used as part of the circuit. The main purpose of the laboratories were to provide the students a better understanding of how each subsystem of the digital communication system, such as the comparator, sampler, modulator, filters works.

During every laboratory session, the authors designed, built, and verified the functionality of each stage using the equipment from B200 lab. Datasheets were also used to check the pin out of each chip used in the design part of the lab. As the project progressed, a schematic diagram were made for each components using PSpice OrCAD Capture. Also, two PIC16F690 chips were used in this lab. The provided code was programmed into the PIC using the MPLAB IDE software available free from Microchip.

  - Accomplishments

Table 1 describes the accomplishment of each of the first five laboratory done for this course. The lab manual provided to the authors at the beginning of the course were used as a reference for the description of the laboratory projects.

| Laboratory Projects | Description |
| --- | --- |
| Interfacing to Infrared Transducers | Understand the functionality of an LED and phototransistor. Design and build a trans-impedance amplifier for the phototransistor. |
| Digital In – Digital Out | Construction of a pseudorandom data generator, a threshold comparator, and a sampling circuit. |
| Modulation and Receiver Front End | Design and characterization of an active band-pass receiver filter. Design of a carrier oscillator and a modulator to provide on-off keying of the transmitter. |
| Envelope Detector | Design and characterization of an envelope detector. End-to-end checkout of the infrared digital communication system. |
| Clock Recovery | Design, construction, test, and characterization of a clock recovery circuit using a microcontroller. |

Table 1. Description of laboratory projects.

- Circuit Documentation

  - Description of the Infrared Digital Communication System Design

A circuit for the transmitter and receiver were built separately on two breadboards. The design of each part will be described in detail. First, the design of the transmitter will be discussed, then the receiver design. A block diagram of both this infrared digital communication system is shown in Figure 1.

Figure 1. The infrared digital communication system

2.2 Transmitter Circuit Design

The transmitter board consists of three main subsystems, a pseudorandom “data” generator, a carrier oscillator and modulator, and an infrared LED, as shown in the block diagram of the transmitter in Figure 1.

- A pseudorandom sequence generator (PSG), in this case, a 16-bit shift register that cycles through all 15 non-zero states, was built using the PIC16F690 microprocessor to simulate “data” to transmit over the channel. The design of the PSG was implemented by programming the provided code into the PIC microcontroller. Another way to do this process would have been using discrete gates. The code for the PIC is attached in the Appendix. The clock frequency, called the “bit clock”, was set at 100Hz square wave using the function generator Tektronix AFG 3022B. This is the external clock input produced to synchronize the pseudorandom sequence output.

- For the design of the 40 kHz carrier oscillator, the 50% Duty Cycle design of the LM555 timer were used. Since this is an optical communication system, a special case of amplitude modulation called “on-off keying” is used. The PSG is used to modulate the square wave carrier produced by the oscillator, so that it is on whenever the binary bit data is a 1, and off when the binary bit is 0. A basic diagram of what is described is shown in Figure 2. The mixer multiplies the signal by 1 to turn the carrier on and multiplies by 0 to turn the carrier off. The mixer is implemented with an AND gate.

Figure 2. Transmitter Circuit

- The modulated carrier is used to drive the LED on and off, with a transistor acting as the driver. Figure 3 shows a more detailed diagram of the transmitter circuit.

Figure 3. More detailed diagram of the transmitter circuit

- In addition to those three main components, a 9 V DC wall transformer was used as an input for an LM7805 voltage regulator to output 5V DC power supply. This 5 V DC helps to power all the subsystems on the transmitter breadboard. Figure 4 shows the diagram of this system.

Figure 4. DC Power Supply for Transmitter

Figure 5. Shows the physically completed transmitter circuit design on a breadboard. In addition, the final schematic diagram of the transmitter circuit is shown in Figure 6.

Figure 5. Transmitter Circuit Breadboard

Figure 6. Final Schematic Diagram of the Transmitter Circuit

2.3 Receiver Circuit Design

The receiver is a more complicated system than the transmitter since it consists of six main subsystems. As shown in the receiver block diagram in Figure 1, it starts with a transducer circuit, which in this case is a phototransistor and an op-amp-based trans-impedance amplifier that functions as a current-to-voltage converter. Then, the received voltage passes through the band-pass filter which is then filtered and amplified. It then goes through the envelope detector, which demodulates the signal. Followed by the threshold comparator, an LM311 configured to produce logic-compatible output voltage levels. The sampling circuit is a D flip-flop. Finally the clock for the flip-flop must be recovered from the received data stream.

- The infrared transducer in the receiver circuit is a phototransistor, which is basically a BJT transistor with special base configuration together with an op-amp trans-impedance amplifier, which convert current to voltage.

- An LM311 is used as a voltage comparator. The binary data received that go through the demodulator will be distorted and noisy, and the voltage levels will not be standard logic levels. Thus, the voltage comparator is used to recover the standard voltage levels.

- The 74LS74A D flip-flop is used to sample the signal. Since the D flip-flop is rising-edge triggered and the PSG is falling-edge triggered, this allows the received signal time to go through the receiver filters and also better timing for sampling.

- Two TL072 op-amps were used to build the second-order dual-amplifier band-pass filter circuit. This is an active filter since it uses active components such as op-amps with no need for inductors. Resistors and capacitors were used to get the desired output. Figure 7 shows the dual amplifier diagram.

Figure 7. Dual-Amplifier Band-pass Filter

The resistors used for R= 1.8kΩ and for R1= 18kΩ, the capacitor value C= 2.2uF

- The envelope detector consists of three stages as shown in the diagram in Figure 8.

- The first stage is a half-wave rectifier made with the op-amp and the diode

- The second stage is a simple low-pass filter made with the resistor R1 and capacitor C1. The bandwidth of this low-pass filter obeys two conditions. It must be much less than the carrier frequency of the envelope detector input signal while it must also be large enough to go through the modulation (envelope).

- The third stage is another low-pass filter using resistor R2 and the capacitor C2. Since the frequency of DC component is zero, the value of C2 is 1uF in order to filter out the DC component.

Figure 8. Envelope Detector

- A PIC16F690 is used for the time recovery subsystem in the receiver circuit. The code provided for the PIC is attached in the Appendix. In most communication systems only the data channel is available and the bit timing must be recovered from the data stream itself.

Figure 9 shows the physically completed receiver circuit on a breadboard. The final schematic of the receiver is shown in Figure 10.

Figure 9. Receiver Circuit Breadboard

Figure 10. Schematic Diagram of the receiver circuit

  - Experimentation and Verification

The experimentation and verification done in each lab project helped with confirming the correct functionality of each stage of the transmitter and receiver circuit. The first stage was to check the correct functionality of the simple optical communication between the LED in the transmitter board and the photoresistor in the receiver board.

Figure 11. The LED-Phototransistor Interface

The phototransistor was characterized in terms of distance and exposure to light. In Graph 1. The resulting characteristic curves of the phototransistor is evidently. The farther the phototransistor is from the LED, the less current it gets, and the less the distance, the more current it gets. In addition, if the phototransistor is covered from the ambient light, it showed almost no current. The current and voltage values were measured using the Agilent 34401A Digital Multimeter from B200 lab.

Graph 1. Characteristic Curves of Phototransistor

A PIC16F690 from Microchip was used to generate the Pseudorandom Data Generator, and the function generator Tektronix AFG 3022B was used to generate a 100Hz square wave bit clock. In Figure 12. A screen capture from the Agilent Technologies DSO7014B Digital Storage Oscilloscope shows the PSG and the bit clock.

Figure 12. Output PSG and bit clock from the transmitter circuit

The voltage comparator was the first subsystem built in the receiver circuit. In Figure 13. The correct performance of the comparator was tested by running a simulated “distorted” sinusoidal signal, which was created with a function generator, through the comparator. The output of the comparator shows a square wave signal synchronized with the input simulated sinusoidal signal. Whenever the sinusoidal amplitude crossed the threshold, the comparator is high, and whenever the sinusoid is lower than the threshold, the comparator output is 0 too.

Figure 13. Comparator output and the input sinusoidal signal from the receiver circuit

Due to the noisy input signal, the comparator output may change state concurrently whenever the input passes through the threshold level. This is shown as noise in at one edge of the comparator output as seen below in Figure 14.

Figure 14. Noise in the comparator output as the noisy input signal passes through the threshold

A D flip-flop was connected with the comparator such that the output of the comparator goes through the D flip-flop to be sampled once per bit time. In Figure 15. a capture of the oscilloscope showing the PSG, comparator output, D flip-flop output, and the bit clock is seen. As was expected, the PSG and the comparator were falling-edge triggered, while the D flip-flop was rising-edge triggered.

Figure 15. Oscilloscope capture of PSG, Comparator output, D flip-flop output, and bit clock

The AND gate and LED driver were added afterwards to the transmitter circuit. In Figure 16, the PSG, AND gate, and LED anode output are shown. The AND gate output and the LED anode waveform have that shape and form because they had been modulated. Multiple oscillations are incorporated in each of the squares.

Figure 16. PSG waveform, AND Gate output, and LED anode waveform from oscilloscope

The dual-amplifier bandpass filter was added to the receiver circuit and its performance was shown in Figure 17. As seen in the figure, all the low and high frequencies from both positive and negative side that were not needed were filtered out.

Figure 17. Oscilloscope screen capture of the PSG, bit clock, LED anode and passband waveforms

The envelope detector was then tested, and then the resulting waveforms were shown in the oscilloscope. A screen capture of the oscilloscope from this stage can be seen in Figure 18. The envelope detector input is the bandpass filter output, the half-wave rectifier eliminates the negative part of the envelope detector input. The envelope output is as a result of allowing only the DC value of the envelope to pass through the filter. The output of this filter has a value that is midway between the voltage corresponding to “1” and the one corresponding to “0”.

Figure 18. Oscilloscope screen capture of the envelope detector input, half-wave rectifier output, envelope output, and threshold

The comparator output and the PSG are the same signal, which means that the transmitted signal is received correctly at the receiver end. The resulting signals are shown in Figure 19.

Figure 19. Comparator signal from receiver and PSG signal from transmitter

Another PIC16F690 was added to create the recovery clock. In Figure 20. The PSG waveform from the transmitter, the D flip-flop output from the receiver, the transmitter bit clock and the recovery clock are shown. The correct functionality of the transmitter-receiver is confirmed since the same shape of the PSG was seen in the receiver D flip-flop with a 5ms delay that was measured in lab.

Figure 20. Oscilloscope screen capture of PSG and bit clock from transmitter and D flip-flop and Recovery clock from receiver

  - Results

The final result is a capture of the oscilloscope showing the PSG output, D flip-flop output, transmitter’s bit clock, and the receiver’s recovered bit clock. This confirmed the proper operation of both final transmitter and receiver circuit since the D flip-flop output is the same signal as the PSG but with a delay which was measured in the lab to be 5ms. Figure 20 shows the screen capture from the oscilloscope.

In addition, 42.5 ~ 44.5 inches was the maximum spacing between the transmitter and receiver where a signal can still be seen in the oscilloscope.

- Conclusion

Throughout the course of five weeks, the authors were able to design and build an optical digital communication system involving a transmitter-receiver circuit.  The fully operation of the transmitter and receiver were verified at the end of the fifth laboratory project.

The authors were able to learn what is involved in designing and building a digital communication system.

- Appendix

Code for creating the Pseudorandom Data Generator for the PIC16F690

// File Name: psg3.asm

// Date: September 2012

// Author: Bruce A. Black

//

//	processor	16F690

//

#include <xc.inc>

psect	bssBANK0, class=BANK0, delta=1, space=1

waitforit:	ds	1

temp:		ds	1

q:			ds	16

savedregs	ds	4

;

psect	text1, class=CODE, delta=2

global _mainpsg

SIGNAT _mainpsg,88

_mainpsg:

; Initialize the device

clrf    STATUS  ;0x03

bsf	RP0  ;0x03,5		STATUS, RP0=1, select bank 1

bcf	RP1  ;0x03,6		STATUS, RP1=0, select bank 1

bsf	IRCF0    ;0x0F,4		OSCCON bit 4 = 1 for 8 MHz

;	Set up port C

bcf	RP0 ;0x03,5		RP0=0, select bank 2

bsf	RP1 ;0x03,6		RP1=1, select bank 2

clrf BANKMASK(ANSEL)	;0x1E	;ANSEL = 0x00, make the IO digital

clrf BANKMASK(ANSELH)	;0X1F	;ANSELH = 0x00

bsf	RP0 ;0x03,5		RP0=1, select bank 1

bcf	RP1 ;0x03,6		RP1=0, select bank 1

bcf	TRISC1    ;0x07,1		TRISC bit 1 = 0, make C1 an output

bcf	RP0 ;0x03,5		RP0=0, select bank 0

bcf	RP1 ;0x03,6		RP1=0, select bank 0

bcf	RC1   ;0x07,1		PORTC, RC1=0

;	Set up for interrupts

bsf	RP0 ;0x03,5		RP0=1, select bank 1

bcf	RP1 ;0x03,6		RP1=0, select bank 1

bcf	INTEDG  ;0x01,6		OPTION_REG, INTEDG=0, falling edge interrupt

bcf	INTF    ;0xB,1		INTCON, INTF=0, clear interrupt flag

bsf	INTE    ;0xB,4		INTCON, INTE=1, enable interrupt

;	initialize variables

bcf	RP0 ;0x03,5		RP0=0, select bank 0

bcf	RP1 ;0x03,6		RP1=1, select bank 0

movlw	0x01

movwf	(q)

movlw	0x00

movwf	(q+1)

movwf	(q+2)

movwf	(q+3)

movwf	(q+4)

movwf	(q+5)

movwf	(q+6)

movwf	(q+7)

movwf	(q+8)

movwf	(q+9)

movwf	(q+10)

movwf	(q+11)

movwf	(q+12)

movwf	(q+13)

movwf	(q+14)

movwf	(q+15)

;	enable interrupt

bsf		GIE ;0x0B,7	INTCON, GIE=1

; This is the main loop

there:	movlw	0x01

movwf	(waitforit)	;Set waitforit=1

;	wait here for an interrupt

here:	movf	(waitforit),f

btfss	ZERO    ;0x03,2	STATUS, Z, Exit loop if waitforit=0

goto	here

;	advance the pseudorandom sequence

movf	(q+0),w

movwf	(temp)

movf	(q+1),w

movwf	(q)

xorwf	(temp),f

movf	(q+2),w

movwf	(q+1)

movf	(q+3),w

movwf	(q+2)

xorwf	(temp),f

movf	(q+4),w

movwf	(q+3)

movf	(q+5),w

movwf	(q+4)

movf	(q+6),w

movwf	(q+5)

movf	(q+7),w

movwf	(q+6)

movf	(q+8),w

movwf	(q+7)

movf	(q+9),w

movwf	(q+8)

movf	(q+10),w

movwf	(q+9)

movf	(q+11),w

movwf	(q+10)

movf	(q+12),w

movwf	(q+11)

xorwf	(temp),f

movf	(q+13),w

movwf	(q+12)

movf	(q+14),w

movwf	(q+13)

movf	(q+15),w

movwf	(q+14)

movf	(temp),w

movwf	(q+15)

;	update the output

movf	(q),f		;Test q(0) for zero

btfss	ZERO    ;0x03,2		STATUS, Z

goto	setone

setzro:	bcf	RC1 ;0x07,1		RC1=0

goto	there		;Go back to top of loop

setone:	bsf	RC1 ;0x07,1		RC1=1

goto	there		;Go back to top of loop

; The interrupt routine

psect	intentry, class=CODE, delta=2

interrupt_function:

bcf		RP0 ;0x03,5		RP0=0, select bank 0

bcf		RP1 ;0x03,6		RP1=0, select bank 0

movwf	(savedregs)

movf	STATUS, w   ;0x03,w		save STATUS

movwf	(savedregs+1)

movf	FSR, w  ;0x04,w		save FSR

movwf	(savedregs+2)

movf	PCLATH, w   ;0x0A,w		save PCLATH

movwf	(savedregs+3)

ljmp	clockedge

;

psect	text2, class=CODE, delta=2

clockedge:

btfss	INTE    ;0xB,4		INTCON, INTE=1?

goto	done

btfss	INTF    ;0xB,1		INTCON, INTF=1?

goto	done

bcf		INTF    ;0xB,1		INTCON, INTF=0

clrf	(waitforit)	;Set waitforit=0

done:

movf	(savedregs+3),w

movwf	PCLATH  ;0x0A		restore PCLATH

movf	(savedregs+2),w

movwf	FSR ;0x04		restore FSR

movf	(savedregs+1),w

movwf	STATUS  ;0x03		restore STATUS

movf	(savedregs),w

retfie

Code for the time recovery for the PIC16F690

//File Name: ClockRecovery.c

//Date:		September 2012

//Purpose: Clock Recovery for UNRZ Using Interrupt-on-Change

//Author: Bruce A. Black

//Modified by:	on:

#include <htc.h>

#pragma config FOSC=INTRCIO, WDTE=OFF, PWRTE=ON, CP=OFF, MCLRE=OFF

#define COMPIN RB5

#define SAMPLECLK RB6

unsigned char waitforit, x;

int count;

void interrupt timertick(void)

{

if ((T0IE == 1)&&(T0IF == 1))	// Timer interrupt

{

T0IF = 0;	// Clear interrupt flag

waitforit = 0;	// Let the main loop proceed

}

if ((RABIE == 1)&&(RABIF == 1))	// Edge detected

{

count = 0;

SAMPLECLK = 0;

x = COMPIN;	// Clear the edge detection

RABIF = 0;	// Clear the interrupt flag

}

return;

}

void main(void)

{

// Set up port B

ANSEL = 0x00; // Make the IO digital

ANSELH = 0x00;

TRISB5 = 1; // Input

TRISB6 = 0; // Output

SAMPLECLK = 0;

IOCB5 = 1;	// Interrupt on change enabled for COMPIN

RABIE = 1;	// Enable register A/B interrupt

x = COMPIN;	// Clear any detected edges

RABIF = 0;	// Clear interrupt flag

// Initialize clock recovery loop

count = 0;

// Set up timer0

T0CS = 0; // Clock source is fosc/4

PSA = 0; // Use prescaler

PS2 = 0; // Prescaler set for 1:2

PS1 = 0;

PS0 = 0;

T0IF = 0; // Clear interrupt flag

TMR0 = 0x00; // Initialize timer value

T0IE = 1; // Enable timer 0 interrupt

// And away we go...

ei();

while (1)

{

waitforit = 1;

while (waitforit) {}  // Wait for timer interrupt

count++;

if (count == 10)

{

SAMPLECLK = 1;

}

if (count == 20)

{

SAMPLECLK = 0;

count = 0;

}

}

}